# Parameter Identification of BLDC

Table of Contents

Introduction	2

Parameter Identification	2

Number of Pole Pairs (P)	2

Phase Resistance (Rph)	3

Direct Measurement techniques	3

Blocked-Rotor DC Test	3

Temperature Compensation	3

Phase Inductance (Lph)	4

Direct Measurement Techniques	4

AC Voltage (Impedance) Method	4

DC Step Response (Transient) Method	4

Back-emf Constant (ke)	5

Open-Circuit (Generator Mode) Test	5

Free-Running (Coasting) Test	5

No-Load Steady-State Test	6

Viscous friction coefficient (B)	6

Algebraic and Graphical Method	6

Moment of inertia (J)	7

Deceleration (Retardation or Coast-Down) Method	7

# Introduction

Parameter identification constitutes a fundamental prerequisite for the accurate mathematical modeling, dynamic analysis, and optimal control of Brushless DC (BLDC) motors.

Primarily, the development of robust and effective control strategies relies heavily on an accurate understanding of the plant model to be controlled. A motor controller can only provide appropriate compensation if the parameters programmed into the control algorithm match the physical conditions of the real motor; otherwise, the controller will likely fail to regulate the system properly.

Furthermore, parameter identification is critical for developing high-fidelity simulation environments that bridge the gap between theoretical design and practical application.

While traditional physics-based models provide a theoretical foundation, they often fail to capture inherent nonlinearities such as viscous friction, dead zones, cogging torque, and load variations under dynamic operating conditions. System identification techniques address this by providing data-driven, empirical models that accurately represent actual steady-state and transient responses[1].

Experimental parameter identification is especially important because manufacturers frequently omit precise parameter specifications from their datasheets, making estimation mandatory for custom drive design.

In addition to control and simulation, accurate parameter estimation plays an indispensable role in system monitoring and fault detection.[2] By identifying and continuously monitoring motor parameters, engineers can establish relations that reflect deviations from nominal operation.

# Parameter Identification

## Number of Pole Pairs (P)

To experimentally determine the number of poles in a Brushless DC (BLDC) motor, the standard methodology relies on analyzing the relationship between the machine's mechanical rotational speed and the steady-state electrical frequency of its induced back-electromotive force (back-EMF). This parameter identification process requires operating the BLDC motor in generator mode.[3] While the rotor is being driven at this constant speed, the stator terminals are kept open-circuited, and the induced open-circuit voltage is measured.

Once the electrical frequency and mechanical speed are acquired, the total number of magnetic poles pairs (P) can be directly computed.

P=120×f2 N

Where f is the frequency of the back-emf in Hz and (N) is the rotational speed of the rotor in RPM.

## Phase Resistance (Rph)

### Direct Measurement techniques

The most straightforward approach involves utilizing an ohmmeter, digital multimeter, or RLC meter to measure the line-to-line resistance directly at the motor terminals. Because a standard BLDC motor consists of three phases, measurements should be taken across all three possible phase terminal pairs (e.g., U-V, U-W, and V-W). The per-phase resistance is then computed by averaging these line-to-line measurements. For instance, the phase resistance can be calculated using the formula, [4]

Rph=Ruv+Rvw+Rwu6

### Blocked-Rotor DC Test

In this experimental procedure, a variable or constant DC voltage is applied across two terminals of the motor. During this test, the rotor must be mechanically locked. By forcing the angular velocity to zero, the motor's back-electromotive force (EMF) is nullified. When the constant DC voltage is applied, the current eventually reaches a steady-state condition where the rate of change of current is zero, allowing the inductive component of the circuit to be safely neglected. The terminal resistance can then be determined using Ohm's law by dividing the applied DC voltage by the measured steady-state current (Rterminal=V/I ). Because the circuit path encompasses two phase windings connected in series, the individual per-phase resistance is exactly half of the measured terminal resistance, [3]

Rph=Rterminal2

### Temperature Compensation

When utilizing the blocked-rotor DC test, it is critical to recognize that the continuous flow of current will cause the stator windings to heat up, which consequently alters the resistance of the copper wire. The accurate phase resistance at an elevated operating temperature (T) can be compensated using the standard temperature-resistance relationship [3]

RT=234.5+T234.5+To×Rph

Where T,and To are measured in Celsius (Co).

## Phase Inductance (Lph)

### Direct Measurement Techniques

The most straightforward approach is to utilize a digital RLC meter or an inductance meter to directly measure the line-to-line inductance across two motor terminals at a specific frequency. Because the magnetic circuit's reluctance can vary slightly depending on the alignment of the rotor's permanent magnets relative to the stator teeth, it is recommended to take these line-to-line measurements at several different rotor angular positions and calculate the average.

Another direct sensor-based approach involves integrating the stator winding into a parallel resonance circuit. By placing the inductive winding in parallel with a known capacitor and providing a digital pulse, the circuit will resonate. A microcontroller can calculate the resonance frequency by measuring the wave's period, thereby allowing the system to derive the inductance mathematically.[4]

Once the average line-to-line inductance is determined, the per-phase inductance is obtained simply by dividing the value by two, assuming a symmetrical star-connected winding.

### AC Voltage (Impedance) Method

Phase inductance can be accurately identified by applying a known, variable AC voltage—typically from a function generator or an AC power supply at a specified frequency (e.g., 60 Hz)—across two terminals of the locked motor. In this configuration, the motor behaves equivalently to a series RL circuit. Because the total impedance comprises both resistive and inductive components, the known total resistance (Rterminal)—previously measured via a DC test—is used to isolate the total inductive reactance (Xterminal) using the formula,

Xterminal=Z2-Rterminal2

Finally, the phase inductance is calculated by isolating it from the reactance formula,

Lph=Xterminal4πf

### DC Step Response (Transient) Method

An alternative electromechanical method evaluates the transient response of the phase current when subjected to a stepwise DC voltage.[5] When a DC voltage (Vdc) is abruptly applied across two phases of a locked motor, the current does not rise instantaneously but instead follows a first-order exponential trajectory defined by ,

it=VdcRterminal1-e-t/τ

where (Rterminal) is the equivalent line-to-line resistance and (τ) is the electrical time constant of the RL circuit.

To find the inductance, the transient current is recorded until it reaches its maximum steady-state value i∞. The electrical time constant (τ) is extracted by finding the exact time it takes for the current curve to reach 63.2% of this steady-state value. Once τ is identified, the equivalent phase inductance is computed using the relationship,

Lph=τ × Rterminal2

## Back-emf Constant (ke)

### Open-Circuit (Generator Mode) Test

In this configuration, the motor shaft is mechanically coupled to a prime mover which drives the rotor at a precisely known, constant rotational speed. During this externally driven rotation, the stator armature terminals are kept completely open-circuited, ensuring that no current flows through the windings. Because the current is zero, the internal voltage drops across the stator resistance and inductance are eliminated, making the induced back-EMF directly equal to the measured terminal voltage.

An oscilloscope or digital multimeter is used to measure the line-to-line RMS voltage across the open phases. To find the individual phase back-EMF, this RMS line voltage is typically divided by 3 to obtain the phase RMS voltage, which is then multiplied by to compute the peak phase voltage 2,[3]

ea=VL-L,rms6

The back-EMF constant (ke) can subsequently be derived by dividing this peak voltage by the electrical or mechanical angular velocity of the rotor (ω),

ke=eaω

### Free-Running (Coasting) Test

A free-running or coasting test can be utilized to measure back-EMF without requiring an external prime mover. The motor is initially energized and driven to a target steady-state speed, after which the power supply is abruptly disconnected, forcing the motor to spin freely by virtue of its own rotor inertia.[5] During this coasting phase, the generated phase voltages can be recorded using phase voltage sensors or a DC-link voltage sensor connected to a digital oscilloscope. The recorded peak-to-peak voltage envelope inherently represents the back-EMF, which decays proportionately with the decelerating rotor speed.

### No-Load Steady-State Test

When neither a prime mover nor a coasting setup is optimal, a standard no-load steady-state test can be employed to estimate the back-EMF. In this approach, a DC voltage is applied to run the motor without any mechanical load attached to the shaft. Due to the absence of a load, the armature current drawn by the motor is extremely minimal. Consequently, it is a valid assumption that the ohmic voltage drop across the stator resistance is negligible, allowing the applied terminal voltage to be approximated as equal to the internal back-EMF (ea=Vin2). [6] By varying the input voltage and recording the corresponding steady-state speeds and terminal voltages, a linear voltage-speed profile is plotted. The slope of this curve yields the back-EMF constant.

Figure 1: Back emf voltage versus angular speed.[7]

## Viscous friction coefficient (B)

To measure the viscous friction coefficient of a brushless direct current (BLDC) motor, one must isolate the velocity-dependent frictional losses from the motor's overall electromechanical dynamics. This is typically achieved by analyzing the motor's behavior under steady-state, no-load conditions where the angular acceleration is zero (dω/dt=0).

### Algebraic and Graphical Method

The most straightforward approach is the steady-state no-load test. When a motor operates at a constant speed without any external mechanical load, the inertial torque component (J. dω/ dt) becomes zero. Consequently, the fundamental mechanical equation simplifies, indicating that the entire electromagnetic torque generated by the stator is used to overcome the internal friction,

B.ω=τe-τf

Where (τe) is the electromagnetic torque and (τf) is the static friction.

The electromagnetic torque (τe) can be determined by multiplying the measured stator current (I) by the motor's torque constant (kt). If static friction is negligible (τf≈0), the viscous friction coefficient (B) can be directly calculated using the equation,

B=kt×Iω

Where (ω) is the steady-state mechanical speed in rad/s.

For greater accuracy, researchers plot a characteristic curve of the mechanical torque against the angular speed across multiple operating points. Because viscous friction is proportional to velocity, the relationship forms a linear graph. The viscous friction coefficient is then identified simply by calculating the linear slope of this torque-speed response.

Figure 2: Mechanical torque versus angular speed.[7]

## Moment of inertia (J)

### Deceleration (Retardation or Coast-Down) Method

Straightforward approach to determine the moment of inertia is the deceleration or retardation test. In this experimental procedure, the motor is driven to a steady-state speed (often just above its rated speed), and then the power supply is cut off, allowing the rotor to coast to a halt. The dynamic first-order differential equation governing the unpowered motor,

0=Bω+Jdωdt+τf

Assuming the static friction (τf) is zero, the previous ODE solution is going to be,

ωt=ωo e-tτm

Where (ωo) is initial rotating speed and (τm) is the mechanical time constant which equals (J/B).

By logging the speed coasting down to zero. The recorded data could be fitted to an exponential curve (y=a exb) and since the viscous friction (B) is known,

J=b . B

[1]	A. I. Abouseda, R. Doruk, A. Emin, and O. Akdeniz, “Modeling, Dynamic Characterization, and Performance Analysis of a 2.2 kW BLDC Motor Under Fixed Load Torque Levels and Variable Speed Inputs: An Experimental Study,” Actuators, vol. 14, no. 8, p. 400, Aug. 2025, doi: 10.3390/act14080400.

[2]	M. A. Khan, D.-Z. Baig, H. Ali, and F. R. Albogamy, “Optimized System Identification (SI) of Brushless DC (BLDC) motor using Data-Driven Modeling Methods,” Sci. Rep., vol. 15, no. 1, p. 8497, Mar. 2025, doi: 10.1038/s41598-025-93444-0.

[3]	A. I. Abouseda, R. O. Doruk, and A. Amini, “Parameter Identification and Speed Control of a Small-Scale BLDC Motor: Experimental Validation and Real-Time PI Control with Low-Pass Filtering,” Machines, vol. 13, no. 8, p. 656, Jul. 2025, doi: 10.3390/machines13080656.

[4]	A. F. Desanti, I. Sidharta, H. Erwantono, D. A. Asfani, H. Suryoatmojo, and M. Wahyudi, “Design of Performance and Parameter Measurement System for Brushless Direct Current (BLDC) Motor,” in 2018 International Seminar on Intelligent Technology and Its Applications (ISITIA), Bali, Indonesia: IEEE, Aug. 2018, pp. 175–179. doi: 10.1109/ISITIA.2018.8710896.

[5]	A. Mukhlisin, E. Rijanto, I. N. Sutantra, M. N. Yuniarto, and A. S. Pramono, “Towards an intelligent reconfigurable BLDC motor controller: Commutation configuration and parameters detection,” presented at the THE 4TH BIOMEDICAL ENGINEERING’S RECENT PROGRESS IN BIOMATERIALS, DRUGS DEVELOPMENT, HEALTH, AND MEDICAL DEVICES: Proceedings of the International Symposium of Biomedical Engineering (ISBE) 2019, Padang, Indonesia, 2019, p. 060003. doi: 10.1063/1.5138364.

[6]	F. Sh. Al-Mahturi, D. V. Samokhvalov, and V. M. Bida, “Parameters identification of a brushless DC motor by specification,” in 2018 IEEE Conference of Russian Young Researchers in Electrical and Electronic Engineering (EIConRus), Moscow: IEEE, Jan. 2018, pp. 558–561. doi: 10.1109/EIConRus.2018.8317159.

[7]	M. N. Abdullah, M. K. Mat Desa, E. A. Bakar, and M. N. Akhtar, “Experimental and numerical studies for parameters identification of direct current motor,” Indones. J. Electr. Eng. Comput. Sci., vol. 27, no. 2, p. 592, Aug. 2022, doi: 10.11591/ijeecs.v27.i2.pp592-600.