# Battery Specifications

# Executive Summary

The following specifications outline a custom-designed LiFePO4 battery pack intended to serve as the primary power source for a Brushless DC Motor (BLDCM) system.

| Nominal Voltage | 51.2V |
| --- | --- |
| Total Capacity | 42Ah ≈2150Wh |
| System Max Continuous Discharge | 60ALimitedbyBMS |
| Cell-Level Max Continuous Discharge | 126A |
| Cell Chemistry | Lithium Iron Phosphate LiFePO4 |
| Expected Life Cycle | >2000Cycle |
| Series/Parallel Configuration | 16S7P |
| Cooling Method | Passive |

# System Requirements

The battery pack is designed to power a BLDCM with the following key specifications:

| Nominal Voltage | 48V |
| --- | --- |
| Rated Speed | 3000 RPM |
| Rated Power | 660W |

The system requires two hours of continuous operation at full load.  Assuming a combined electromechanical conversion efficiency (inverter and motor) of 80%, the required energy capacity is calculated as follows:

Ereq=660W80

Consequently, the required capacity in Ampere-hours (Ah) at the nominal system voltage is:

Ahreq=1650 Wh48V≈34.38 Ah

# Battery Design

## Chemistry Selection

Lithium Iron Phosphate (LiFePO4) was selected based on the following engineering parameters:

- Thermal Stability: Inherently stable chemistry minimizes the risk of thermal runaway events.

- Cycle Life: Capable of >2000 charge/discharge cycles, providing a longer operational lifespan compared to alternative lithium chemistries.

- Environmental Impact: Free of toxic heavy metals.

- Environmental Tolerance: Maintains reliable performance across a wide operating temperature range.

- Total Cost of Ownership: Extended lifespan and reduced maintenance requirements yield long-term cost efficiency.

## Cell Specifications

| Form Factor | 32700 |
| --- | --- |
| Nominal Voltage | 3.2V |
| Nominal Capacity | 6 Ah |
| Max Continuous Discharge Current | 18 A |

## Electrical Design

To size the battery pack appropriately, the required capacity must account for the safe Depth of Discharge (DoD) limit of the cells. Assuming a strict DoD limit of 90% (0.9) to preserve cell longevity:

Ahpack=AhreqDoDlimit

Ahpack=34.38 Ah0.9=38.2 Ah

Parallel Configuration (Np):

Given each cell's capacity of 6 Ah, the required number of parallel cells is:

Np=38.26=6.37

Rounding up to ensure adequate capacity, Np is designated as 7.

Series Configuration (\mathbitN\mathbits):

The motor's nominal voltage requirement is 48 V. Based on the 3.2 V nominal voltage of each cell:

Ns base=483.2=15

While 15S yields exactly 48V, a 16S configuration is standard for 48V-class systems. Increasing Ns to 16 accounts for voltage sag under full load, compensates for resistive losses across system wiring and inverter switching.

Final Pack Specifications (16S7P):

| Nominal Voltage | 16×3.2 V=51.2 V |
| --- | --- |
| Nominal Capacity | 7×6 Ah=42 Ah (yielding ≈2150 Wh) |
| Max Continuous Discharge Current | 7×18 A=126 A |

Implementation Details

The final physical assembly utilizes the designed 16S7P configuration using EPRO 32700 LiFePO4 cells.

Battery Management System (BMS):

The pack is governed by a JIABAIDA DB24SA03 BMS.

- Hardware Capabilities: Supports up to 24 series strings.

- Current Limiting: Features a maximum continuous current rating of 60 A. Note: This acts as the primary system bottleneck; the pack cannot output the cell-capable 126 A without triggering over-current protection. However, 60 A is highly sufficient for the continuous ≈18 A draw of the target load.

- Telemetry & Communication: Features Bluetooth connectivity for mobile application monitoring and a UART port for external communication protocols.